Документ №1:

Table of Contents

1	Introduction	7

1.1	Purpose	7

1.2	Services Strategy Development	8

1.3	Audience	9

1.4	Scope	10

1.5	Assumptions	11

2	NIP – IP NGN Design	12

2.1	Overview	12

2.1.1	Core/Distribution Layer	13

2.1.2	Intelligent Edge	16

2.1.3	Aggregation Layer	19

2.1.4	Access Layer	21

2.1.5	Subscriber Edge	27

2.2	Availability	29

2.2.1	Fast Convergence of Network Protocols	31

2.2.2	MPLS Traffic Engineering (TE)—Fast Reroute (FRR) Link and Node Protection	32

2.2.3	Incremental SPF, BFD, IP Event Dampening and OSPF Optimizations	38

2.2.4	Stateful Switch-Over (SSO)/Non-stop Forwarding (NSF)	39

2.2.5	Multicast Convergence	40

2.3	Sizing	40

2.3.1	Subscriber Dimensioning	41

2.3.2	Number of Aggregation Nodes per Neighborhood	44

2.3.3	Number of Distribution Nodes per Neighborhood	44

2.3.4	Purpose of the PoPs	44

2.4	Quality of Service	49

2.4.1	Definition and Requirement	49

2.4.2	Implementation Steps	50

2.4.3	Packet Classification and Marking	52

2.4.4	Trust Boundaries	60

2.4.5	MPLS Service Provider Design	61

2.5	Aggregation Bandwidth Calculations Access	67

2.6	Core Bandwidth Calculations	69

3	Carrier Ethernet Services	72

3.1	Residential Services Architecture	72

3.1.1	Residential Services Model	72

3.1.2	Residential Services	75

3.1.3	Residential SDU/MDU Connectivity Architecture	75

3.1.4	Residential SDU	75

3.1.5	Residential MDU	77

3.1.6	Residential High Speed Internet Access (HSI)	80

3.1.7	Voice Over IP  (VoIP)	83

3.1.8	Video Service – IPTV	86

3.2	Business Service Architecture	88

3.2.1	Business Services MDU Model	90

3.2.2	Business Internet Access	92

3.2.3	Layer 2 Business Services	93

3.2.4	Layer 3 Business Services	96

3.3	Wholesale Services Architecture	98

3.4	Optional Circuit Emulation over Packet (CEoP)	99

3.4.1	Requirement of Traditional TDM Circuits	99

3.4.2	Proposed Design for CEoP	100

4	Appendix A	103

5	Appendix B	105

6	Glossary	108

7	About This Document	114

7.1	History	114

7.2	Review	114

7.3	Document References	114

8	Document Acceptance	116

Table of Figures

Figure 1-1: Skolkovo Overall Technical Architecture	7

Figure 1-2: Layered Services Approach	9

Figure 2-1 – Typical NIP NGN Architecture	12

Figure 2-2 – Core/Distribution Elements at Primary POP	13

Figure 2-3 – Typical Core Network Architecture	14

Figure 2-4 – BGP Route Reflector Architecture	16

Figure 2-5 – Logical Placement of Intelligent Edge Elements	17

Figure 2-6 – Intelligent Edge Elements at Primary POP	18

Figure 2-7 – Depiction of Aggregation Layer	19

Figure 2-8 – Depiction of Access Layer	22

Figure 2-9 – Conceptual Design of Aggregation and Access Network	23

Figure 2-10 – Depiction of Subscriber Edge	27

Figure 2-11 – MPLS Concept	33

Figure 2-12 – MPLS Operation	34

Figure 2-13 – FRR Link Protection	36

Figure 2-14 – FRR Node Protection	37

Figure 2-15 – FRR Bandwidth Protection	38

Figure 2-16 – Multicast Total Convergence	40

Figure 2-17 – Multicast Convergence as a Function of Unicast Convergence	40

Figure 2-18 – Primary PoP Logical Layout	45

Figure 2-19 – Secondary POP Logical Layout for MDU Areas	46

Figure 2-20 – Secondary PoP Logical Layout for SDU Area	47

Figure 2-21 – Tertiary PoP Logical Layout and Co-location Options	48

Figure 2-22 – Class Service 802.1Q/p Field	53

Figure 2-23 – EXP Field MPLS Header	54

Figure 2-24 – IP Header TOS Byte and IP Precedence Bits	54

Figure 2-25 – IP Header TOS Byte and DSCP Bits	56

Figure 2-26 – IP Header DS Field and DSCP PHBs	57

Figure 2-27 – Mapping between QoS Fields at all Layers	59

Figure 2-28 – Modified RFC4594 Marking Values	60

Figure 2-29 – Location of QoS Marking and Policing in SP MPLS Network	61

Figure 2-30 – Three Modes of MPLS/DiffServ Marking for SP	62

Figure 2-31 – Legacy WAG Model for Enterprises	63

Figure 2-32 – Any-to-Any/Full-mesh MPLS VPNS Model for Enterprises	63

Figure 2-33 – Simple 5 Class QoS Model	64

Figure 2-35 – U-PE Bandwidth Projection for 20 Gbps Uplinks (Redundant 10 Gbps)	68

Figure 2-36 – U-PE Bandwidth Projection for 10 Gbps Uplinks (Single 10 Gbps)	69

Figure 2-37 – PE-AGG BW Projection for 40 Gbps Uplinks (Redundant 20 Gbps)	70

Figure 2-38 – PE-AGG Bandwidth Projection for 20 Gbps Uplinks (Single 10 Gbps)	71

Figure 3-1 – Residential Service Architecture	73

Figure 3-2 – Villa SDU Connectivity – HAG Model	76

Figure 3-3 – Villa SDU Connectivity – HAG+POE Switch Model	77

Figure 3-4 – Residential MDU Connectivity	78

Figure 3-5 – Residential HSI Service Delivery Architecture	81

Figure 3-6 – Residential Services: Active/Backup Access Node Redundancy – Option 1	82

Figure 3-7 – Residential Services: Active/Backup Access Node Redundancy – Option 2	83

Figure 3-8 – Residential VoIP Service Delivery Architecture	84

Figure 3-9 – Residential Voice Service Redundancy	85

Figure 3-10 – Residential IPTV Service Delivery Architecture	87

Figure 3-11 – Business Services – MSE Approach	88

Figure 3-12 – Business Services – Aggregation Approach	89

Figure 3-13 – Business Building Architecture	91

Figure 3-14 – Business Internet Service Delivery Architecture	93

Figure 3-15 – E-LINE Service Delivery Architecture	93

Figure 3-16 – E-LAN Service Delivery Architecture	94

Figure 3-17 – Resiliency with Routed CPE	95

Figure 3-18 – Resiliency with Bridged CPE	96

Figure 3-19 – L3VPN Service Delivery Architecture	97

Figure 3-20 – Wholesale Service Architecture	99

Figure 3-21 – Traditional TDM Circuit Delivery	100

Figure 3-22 – Design for Optional CEoP Solution	101

List of Tables

Table 2-1 - Last Mile Fiber Core Count Recommendations	26

Table 2-2 – Summary Skolkovo Demographic Data	41

Table 2-3 – Average Aggregate Residential Bandwidth Estimates	42

Table 2-4 – Similarity Between L2 CoS, IPP and MPLS EXP Bits	55

Table 2-5 – The AF DSCP Values	58

Table 2-6 – Voice Bandwidth with Layer-2 Overhead	66

Документ №2:

Table of Contents

1	Introduction	8

1.1	Purpose	8

1.1.1	Services Strategy	9

1.2	Audience	10

1.3	Scope	10

1.4	Assumptions	11

1.4.1	Security	11

1.4.2	Availability	11

1.4.3	Fault and Error Tolerance	12

1.4.4	Performance and Scalability	12

1.4.5	Interoperability and Openness	12

1.4.6	Reuse and Shared Services	12

1.4.7	Data Capture	12

1.4.8	Agility and Flexibility	13

1.4.9	Buy or Build	13

1.4.10	Management and Support	13

1.4.11	Accessibility and Usability	13

1.4.12	Citizen and business Service Centric	13

1.4.13	Completeness	14

1.4.14	Other	14

2	Business Layer Architecture	16

2.1	Framework	16

2.1.1	Enterprise Business Process Architecture	18

2.1.2	Enterprise Business Component Architecture	21

2.1.3	Class and Composite Application Framework	23

3	Access and Presentation Layer Architecture	25

3.1	Structure	25

4	Applications Layer Architecture	31

4.1	Development	33

5	Data Layer Architecture	35

5.1	Business Context: Metadata Management	36

5.2	Data Exchange: Core Data Services	38

5.3	Data Structure: Data Stores	38

6	Integration Layer Architecture	40

6.1	Business Event and Service Choreography	43

6.2	Enterprise Service Bus (ESB)	44

6.3	Adapters	44

6.4	Integration Versus Interoperability	46

6.5	SCO End to End Integration Architecture	47

7	Infrastructure Layer Architecture	48

7.1	Data Centre	50

7.1.1	Core	52

7.1.2	Aggregation	53

7.1.3	Access	57

7.1.4	Storage	62

7.1.5	Virtualization – Storage, Compute Power and Security	64

7.1.6	Distributed Design Topology and Redundancy	66

7.1.7	Network Management	70

7.1.8	Testing	72

7.1.9	Capacity Planning and Growth	72

8	Security Layer Architecture	74

8.1	Business Architecture Security Component (BASC)	75

8.2	Technical Architecture Security Component (TASC)	76

8.2.1	Data Model	76

8.2.2	Systems Model	78

8.2.3	Infrastructure Model	80

8.2.4	Control Model	81

9	Operations Layer Architecture	83

9.1	Managing Converged Network Industry Best Practices	83

9.2	The TAM Model	83

9.2.1	Implementation NGOSS	84

9.3	ONM Blueprint - SCO	84

9.4	NMS Features and Functions	85

9.4.1	Domain Management	85

9.4.2	Manager of Managers (MoM)	86

9.4.3	Configuration Management	88

9.4.4	Capacity Planning	89

9.4.5	Inventory Management	90

9.5	OSS Features and Functions	91

9.5.1	Service Problem Management Application	91

9.5.2	Performance Management	92

9.5.3	Service Inventory Management	93

9.5.4	Service Provisioning	93

9.5.5	Service Control Platform	94

9.6	BSS Features and Functions	95

9.6.1	Customer Relationship Management (CRM)	95

9.6.2	SLA Management Application	97

9.6.3	Billing System	98

9.7	High Level Design Functional Interactions	98

9.8	Out-of-Band (OOB) Management	99

9.9	Resiliency	99

9.9.1	High Availability	100

9.9.2	Geographical Redundancy	100

10	ONM Overview	101

10.1	OSS/BSS Sample Product Features and Functions	102

11	ONM - Network Infrastructure Provider (NIP)	105

11.1	Architecture	105

11.2	NMS/OSS/BSS Products Features and Functions	106

11.2.1	Cisco Products Features	107

11.2.2	3rd Party Product Vendor Suggestions	108

11.2.3	EMS (Resource Management) Functions	109

11.2.4	NMS (Resource Management) Functions	109

11.2.5	OSS (Service Management) Functions	110

11.2.6	BSS (Customer Relationship Management) Functions	110

12	ONM - Passive Infrastructure Provider (PIP)	111

12.1	Architecture	111

12.2	NMS/OSS/BSS Products Features and Functions	112

12.2.1	Cisco Products Features	113

12.2.2	3rd Party Product Vendor Suggestions	114

12.2.3	EMS (Resource Management) Functions	114

12.2.4	NMS (Resource Management) Functions	114

12.2.5	OSS (Service Management) Functions	115

12.2.6	BSS (Customer Relationship Management) Functions	115

13	ONM - Regulated or Retail Service Provider	116

13.1	Architecture	116

13.2	NMS/OSS/BSS Products Features and Functions	117

13.2.1	117

13.2.2	Cisco Products Features	117

13.2.3	3rd Party Product Vendor Suggestions	118

13.2.4	EMS (Resource Management) Functions	118

13.2.5	NMS (Resource Management) Funcitions	118

13.2.6	OSS (Service Management) Functions	119

13.2.7	BSS (Customer Relationship Management) Functions	119

14	Glossary	120

15	About This Document	124

15.1	History	124

15.2	Review	124

15.3	Document References	124

16	Document Acceptance	125

Table of Figures

Figure 1-1: Skolkovo Overall Technical Architecture	8

Figure 1-2: Layered Services Approach	9

Figure 1-3: SCO Recommended Enterprise Architecture	11

Figure 2-1: Usage of Application Framework For Business Needs	17

Figure 2-2: Illustration Of EBPA Structure	18

Figure 2-3: Categorisation of EBCA Components	21

Figure 2-4: EBCA Example	23

Figure 2-5: Composite Application Framework Example	24

Figure 2-6: Application Framework for Business Needs	24

Figure 3-1: Access and Presentation Components	25

Figure 3-2: Access and Presentation Layer Devices and Media	26

Figure 3-3: Access Channel and Delivery Channels Usage Example	27

Figure 3-4: Cost Per Transaction vs. Transaction Volumes	28

Figure 3-5: N-Tier Application Architecture	29

Figure 4-1: Applications Layer Architecture Example	33

Figure 4-2: Application Transformation	34

Figure 5-1: Entities and Relationships	37

Figure 6-1: Three Processes - Service Oriented Architecture	40

Figure 6-2: Integration Layer Architecture	42

Figure 6-3: Users and Services Within SOA	42

Figure 6-4: Typical Portal Integration	45

Figure 6-5: Message Oriented Middleware as Integration Broker	46

Figure 6-6: SCO End to End Architecture Integration Architecture	47

Figure 7-1: SCO Infrastructure Layer Architecture with Data Centre	49

Figure 7-2: SCO Data Centre Infrastructure Tiered Architecture	50

Figure 7-3: SCO Data Centre Infrastructure Tiered Architecture	51

Figure 7-4:  Logical Flow Traffic through DC Core	53

Figure 7-5: Aggregation Block	54

Figure 7-6:  Virtual Device Context	55

Figure 7-7: Load Balancing Appliance Logical Flow	56

Figure 7-8 – Firewall Appliance Logical Flow	57

Figure 7-9 – Legacy Data Centre Multi Port Model	58

Figure 7-10 – Legacy and Unified Server Connectivity Models	58

Figure 7-11 – Fiber Channel over Ethernet (FCoE)	59

Figure 7-12 – Unified I/O Design – Option 1	60

Figure 7:13 – Unified I/O Design – Option 2	61

Figure 7-14 – Different Network Islands Virtualized To Common Data Centre Infrastructure	65

Figure 7-15 – Proposed Design Showing Options for DR	67

Figure 7-16 – DR Solution using GSS	69

Figure 7-17 – Global Site Selection Process	70

Figure 7-18 – Out of Band Network Management	71

Figure 8-1 : Security Architecture Framework	75

Figure 8-2: System, Control and Infrastructure Models	77

Figure 8-3: Network Layer (Systems Model)	78

Figure 8-4: Operating Systems Layer (Systems Model)	79

Figure 8-5: Application Layer (Systems Model)	79

Figure 9-1: TAM Framework	84

Figure 9-2: ONM Functional Blueprint	85

Figure 9-3: ONM Functional Interactions	99

Figure 9-4:  High Availability Architecture ONM Applications	100

Figure 10-1: SCO High Level  ONM Architecture	102

Figure 10-2: Sample SCO Vendor  Products and Solutions	103

Figure 11-1: High Level NIP ONM Architecture	106

Figure 11-2: Smart City NIP Products and Solutions	107

Figure 12-1: High Level PIP ONM Architecture	112

Figure 12-2: PIP Products and Solutions	113

Figure 13-1: High Level Typical RGSP/RSP ONM Architecture	116

Figure 13-2: Typical RGSP/RSP Products and Solutions	117

List of Tables

Table 8-1: Security Services and Mechanisms	82

Table 10-1: OSS Applications Functions	104

Table 10-2: BSS Applications Functions	104

Table 11-1 – EMS Applications Functions	109

Table 11-2 – NMS Applications Functions	109

Table 11-3 – OSS Applications Functions	110

Table 11-4 – BSS Applications Functions	110

Table 6-1 – EMS Applications Functions	114

Table 12-2 – NMS Applications Functions	114

Table 12-3 – OSS Applications Functions	115

Table 12-4 – BSS Applications Functions	115

Table 13-1 – EMS Applications Functions	118

Table 13-2 – NMS Applications Functions	119

Table 13-3 – OSS Applications Functions	119

Table 13-4 – BSS Applications Functions	119

Документ №3:

Table of Contents

1	Introduction	7

1.1	Purpose	7

1.2	Services Strategy Development	8

1.3	Audience	9

1.4	Scope	10

1.5	Assumptions	10

2	IP Voice Access Solution	14

2.1	Voice Architecture	14

2.1.1	Media Gateway (MGW or MG)	15

2.1.2	Session Boarder Controller (SBC)	16

2.1.3	Signalling Gateway (SG)	16

2.1.4	Class 4 Softswitch for Citywide Voice Solution	17

2.1.5	Class 5 Softswitch for Residential Customers	18

2.1.6	Softswitch for Large, Medium and Small Businesses	19

2.2	IP-Multimedia Subsystem Architecture	19

2.2.1	IMS Call Control Plane	20

2.2.2	IMS Voice Services and Application Layer	22

2.2.3	Voice Network Infrastructure Layer	22

2.3	IMS Reality Check	23

2.3.1	IMS Scope for Access Provider	23

2.4	Access Provider Voice Architecture	24

2.4.1	Growing Call Volumes Demand Scalability	26

2.4.2	Telephony Grade Reliability	26

2.4.3	Introducing New Services	26

2.4.4	Operational Expense Reduction	26

2.4.5	Hosted Call/Help Center	27

2.4.6	Function of SBC	27

2.5	PSTN and VoIP Interconnect	28

2.6	Mobile Interconnect Solution	29

2.7	Introduction to Lawful Interception (LI) Concept	30

2.7.1	Technical Description of LI	31

2.7.2	Lawful Intercept Architecture for Voice Provider	33

2.8	Voice Network Sizing	35

2.8.1	Voice Calculations Assumptions	37

3	Internet Access Solution	40

3.1	Internet Traffic Access Within Regulated Providers	40

3.1.1	Availability	41

3.1.2	Scalability	41

3.1.3	QoS	41

3.1.4	Security	42

3.2	Traffic Peering Architecture	42

3.3	Internet Network Sizing	43

4	IP TV Solution	46

4.1	Overview	46

4.1.1	Architecture	48

4.1.2	Service Components	49

4.2	Video Sites	50

4.2.1	Video Data Center	50

4.2.2	Super Head-End (SHE)	51

4.2.3	Regional Head-End (RHE)	51

4.2.4	Video Hub Office (VHO)	51

4.2.5	Video Switching Office (VSO)	52

4.3	Solution Capabilities	52

4.3.1	Broadcast Video	52

4.3.2	Video on Demand (VoD)	52

4.3.3	High Definition (HD) Video	53

4.3.4	Network Personal Video Recorders (n-PVR)	53

4.3.5	Ad-Insertion	53

4.3.6	Business Oriented Video, Data and Voice Services	53

4.3.7	Future Applications	53

5	Delivery	55

5.1	Broadcast Head-End	55

5.2	Middleware	56

5.2.1	Electronic Program Guide (EPG)	58

5.2.2	Video-on-Demand (VOD) Back Office	58

5.2.3	Asset Management	58

5.2.4	Session Management	58

5.2.5	Entitlement System	59

5.2.6	Navigation Server	59

5.2.7	Video-on-Demand (VoD) Server	60

5.2.8	Conditional Access Broadcast Video and VoD	60

5.2.9	Set-Top-Box (STB) Sub-System	62

5.2.10	Additional Components	64

5.3	Video Service Flows	66

5.3.1	Broadcast Video Service Flow	66

5.3.2	Video-on-Demand Service Flow	67

5.4	Channel Change Time and Video Quality Monitoring	69

5.5	Network Design for IPTV	70

5.5.1	Multicast	70

5.5.2	Quality of Service	72

5.6	High Availability for IPTV	75

5.7	Security for IPTV	75

5.7.1	Reconnaissance	75

5.7.2	Distributed Denial of Service (DDoS) Attacks	76

5.7.3	Hardware-Based Rate Limiters	76

5.7.4	Control Plane Policing	76

5.7.5	Spoofing	77

5.7.6	Authentication and Authorization	77

5.7.7	Subscriber Traffic Isolation	77

5.8	IPTV Solution Sizing	78

5.9	Additional IPTV Design Recommendations	81

5.9.1	CDS Design Recommendation	81

5.9.2	VQI Design Recommendation	82

5.10	IPTV Service Deployment Models for NIP/IPTV Provider	82

5.10.1	NIP/IPTV Provider as Transport Provider - Using Native Multicast with Inter-AS Support	82

5.10.2	NIP as Transport Provider - Using Multicast VPN (mVPN)	84

5.11	IPTV Provider - Aggregating Content from Various Content Providers	87

5.12	IPTV Operations and Management Components	89

5.12.1	Video Assurance Management Solution (VAMS)	90

5.12.2	Performance Management	94

5.13	Summary of Feature Matrix for IPTV	95

5.13.1	Access network	95

5.13.2	Aggregation network	96

5.13.3	Distribution and Core networks	96

6	Quality of Service	97

6.1	Sample QoS Model	97

7	Glossary	101

8	About This Document	108

8.1	History	108

8.2	Review	108

8.3	Document References	108

9	Document Acceptance	109

Table of Figures

Figure 1-1: Skolkovo Overall Technical Architecture	7

Figure 1-2: Layered Services Approach	9

Figure 2-1 - Conceptual Design of Access Provider Service Provider Network	14

Figure 2-2 – Signalling Gateway Functionality: Backhauling of MTP3 from the SS7 Network	17

Figure 2-3 – Class 4 Softswitch Protocol Support	18

Figure 2-4 – Generic IMS Architecture	20

Figure 2-5 –Scope For IMS Voice Provider	24

Figure 2-6 – Proposed Voice Architecture for Voice Provider	24

Figure 2-7 – Access Provider PSTN and VoIP Interconnect	29

Figure 2-8 – Access Provider Interconnect with PLMN	30

Figure 2-9: Concept of Lawful Intercept	31

Figure 2-10 – Voice Provider Lawful Intercept on SBC for VoIP SP Interface	34

Figure 2-11 – Voice Provider Lawful Intercept on MGW for PSTN SP Interface	35

Figure 3-1 – Depiction of Traffic Peering Blocks	40

Figure 4-1 – IPTV High Level Architecture	47

Figure 4-2 – IPTV Provider Solution Design	50

Figure 5-1 – Logical Relationship of IPTV Components	55

Figure 4-4 – Content Encryption	62

Figure 4-5 – CDS Solution	64

Figure 4-6 – VQI Solution	66

Figure 5-5 – Broadcast Video Flow	66

Figure 7-8 – VoD Flow	68

Figure 7-9 – Channel Change Even Summary	69

Figure 7-10 – Network Design Aspects of IPTV	70

Figure 7-11 – IGMP Static Join	72

Figure 7-12 – Transport Provider using Native IP Multicast – Option– A	84

Figure 7-13 – Transport Provider using Multicast VPN (mVPN) – Option– B	85

Figure 7-14 –Multicast VPN	86

Figure 7-15 –Default MDT in mVPN	87

Figure 7-16 –Data MDT in mVPN	87

Figure 7-17 –RSSP as IPTV Service Provider – Option-C	89

Figure 7-18 –Multicast Manager	92

Figure 7-19 – IPTV Information Center Service Dashboard	93

Figure 4-1 – Simple 5 Class QoS Model	98

List of Tables

Table 1-1: Average Aggregate Bandwidth Estimates - Skolkovo Residential Subscribers	12

Table 2-1 - Phone lines Estimations Per Phase	36

Table 2-2 - Assumptions Used To Estimate Phone Lines	37

Table 2-3 – Voice Network Initial Design Parameter	38

Table 3-1 - Internet Connections and Yearly Bandwidths Expectations	44

Table 3-2 –Expected Yearly Internet Connection Bandwidths by Customer Type	45

Table 4-1 – Content Encryption Message	62

Table 5-2 - IPTV Network Initial Design Parameter	78

Table 5-3 - Number of Business Expected in the	80

Table 5-4 – Calculation of Maximum Possible STBs Per Phase	80

Table 5-5 - IPTV Network Elements for Deployment Option-A	84

Table 5-6 - IPTV Network Elements for Deployment Option-B	86

Table 5-7 - IPTV Network Elements for Deployment Option-C	89

Table 6-1 – Voice Bandwidth with Layer-2 Overhead	99

Документ №4:

Contents

1	Introduction	6

1.1	Project Description	6

1.2	Document Purpose and Usage	6

1.3	Document Scope	6

1.4	Assumptions and Caveats	7

2	Telecommunication Spaces	8

2.1	Telecom Entrance Facility	8

2.1.1	Entrance Facility Requirement	8

2.1.2	Entry Boxes or Hand Holes/Maintenance Holes	8

2.1.3	Entry Pipes (Lead-In Ducts)	10

2.1.4	Termination/Distribution Point in Building	10

2.2	Telecommunications Room	11

2.2.1	Main Telecom Room	11

2.2.2	Floor Telecom Room	12

2.2.3	Telecom Enclosure	14

2.2.4	Apartment Consolidation Point (ACP)	14

2.2.5	Villa Distribution Cabinet	15

2.3	Telecom Equipment Room - General Specifications and Requirements	15

2.3.1	General Design Considerations	15

2.3.2	Raised-Floor Specifications	16

2.3.3	Door Requirements	16

2.3.4	Civil Services	17

2.3.5	Electrical Power	17

2.3.6	Environmental Requirements	17

2.3.7	Electromagnetic Interference	18

2.3.8	Lighting Requirements	18

2.3.9	Fire Suppression System	19

2.3.10	CCTV / Monitoring System	20

2.3.11	Uninterrupted Power Supply	20

2.3.12	Acoustic Noise	20

3	Structured Cabling System	21

3.1	Horizontal Distribution System	22

3.1.1	Horizontal Pathways and Cable Containment Capacities	22

3.1.2	Horizontal Conduits and Spacing	25

3.1.3	26

3.1.4	Horizontal Cabling	26

3.2	Work Area	30

3.3	Backbone Cabling System	33

3.3.1	Risers from Main Telecom Room to Individual Floors	33

3.3.2	Backbone Cable General Consideration	34

3.3.3	Cable Containment, Routing, and Installation	36

3.3.4	Vertical Backbone Cable Type	37

3.4	Fibre Backbone Infrastructure Design Criteria	38

3.5	Grounding and Electrical Protection	39

4	No Objection Certificate Requirements	42

4.1	Design and Construction	42

4.2	Material No Objection Certificate	42

4.3	Site Inspections	43

4.4	Handover and Acceptance	43

4.5	NOC Validity	43

5	Appendix-1	44

6	Appendix-2	45

7	Appendix-3	46

8	Appendix-4	47

9	Appendix-5	48

10	Appendix-6	49

11	Glossary	52

12	About This Document	58

12.1	History	58

12.2	Review	58

12.3	Document References	58

13	Document Acceptance	59

Table of Figures

Figure 1: Entrance Box Representation	9

Figure 2: Typical Fire Stop Representation	19

Figure 3: Conduit Usable Area and Factors Affecting It	23

Figure 4: Pictorial Representation of Horizontal Pathways and Containment Systems	24

Figure 5: Pictorial Representation - Conduits	25

Figure 6: Horizontal Cabling for Retail Outlet	30

Figure 7: MDU Apartment Cabling Scenario	30

Figure 8: Pin/Pair Assignment	31

Figure 9: CRE Components Cabling	32

Figure 10: Typical Sleeve and Slot Installations	33

Figure 11: FTTH Architecture for Group of Towers	35

Figure 12: Tight Buffered Fibre Optic Cable	39

Figure 13: Scope of Standard 607 for Telecom Grounding	40

Figure 14: Example of Bonding as Per EIA-607	41

Figure 15: JRC-12 Details	46

Figure 16: Typical MTR Size 5 x 4 Sq. Meter	47

Figure 17: Floor Layout – Typical FTR (2.5 x 2.5 meter)	48

List of Tables

Table 1: Entrance Facility Requirements Per Building Type	9

Table 2: MTR Minimum Size Requirement	11

Table 3: Minimum TR Sizes	12

Table 4: TER Sizing Requirements	16

Table 5: Power Calculation – TER	18

Table 6: TER Contaminant Thresholds	18

Table 7: Maximum Capacity Containment- U/UTP Cat6 Cables	23

Table 8: Conduit Size and Cat6 Cable Quantity	26

Table 9: Cable Types for Horizontal Cabling	27

Table 10: Specification of Cat6 Cable	28

Table 11: Specs for Multilayer Cable Trays for Vertical Risers with 40% Fill Ratio	36

Table 12: Minimum Separation Distance from Power Source as Per Standard 569	36

Table 13: Supported Distance - Fibre Cable	37

Table 14: Exact Power Calculation for TER	45

Документ №5:

Table of Content

1	Introduction	7

1.1	Purpose	7

1.2	Audience	7

1.3	Scope	7

1.4	Assumptions	8

2	OSP Guideline	9

2.1	Overview of Communications Master Plan	9

2.2	OSP Design Overview	9

2.2.1	Definition of POPs	10

2.2.2	Definition of Data Center	12

2.3	Plot Location for Communications Network POPs	13

2.3.1	Manhole and Conduit Layout	16

2.3.2	Manholes and Duct-bank	17

2.4	Metro Backbone and Design Considerations	21

2.4.1	Smart City OSP Design Process	21

2.4.2	Fiber Design Architecture	22

2.4.3	Fiber Count Recommendations	22

2.4.4	Fiber Optic Specification	23

2.4.5	High-Capacity/Density Fiber-Optic Panel and Modular Shelves	26

2.4.6	Splice Tray	27

2.4.7	Splicing Hardware	28

2.4.8	Low Profile, High Density Fiber Patch Panels	28

2.4.9	LC Connector	29

2.4.10	Fiber Splicing Implementation Recommendations	30

2.5	OSP Intelligent Physical Infrastructure Management	31

2.5.1	Introduction	31

2.5.2	Architecture of Intelligent Infrastructure Management	33

2.5.3	Optical Network Management System (ONMS)	35

2.6	Applications	42

2.7	Pathway System	43

2.7.1	Ducts Backbone and Design Considerations	43

2.7.2	Manholes-Handholes	47

2.7.3	Other Important Design Considerations for Manholes	51

2.8	Ducts (Conduit) and Duct bank	52

2.9	Communications Media	53

2.10	OSP Apparatus	54

2.11	Midspan Access	55

2.12	Entrance Facility	59

3	Inside Plant (ISP) Recommendations for DC and POP	60

3.1	Aims and Objectives	60

3.2	Design Overview Acceptance Criteria	60

3.3	Cable Containment, Routing and Installation	61

3.4	Horizontal Office Areas	65

3.5	Cable Tray and Accessories Installation	65

3.6	Trunking and Accessories Installation	66

3.7	Communications Network Equipment Room Specifications	66

3.8	Data Center Tier Types	67

3.8.1	Tier I Data Center – Basic	67

3.8.2	Tier II Data Center – Redundant Components	67

3.8.3	Tier III Data Center – Concurrently Maintainable	67

3.8.4	Tier IV Data Center – Fault Tolerant	68

3.9	Telecommunications Tier Types	68

3.9.1	Tier I (Telecommunications)	68

3.9.2	Tier II (Telecommunications)	69

3.9.3	Tier III (Telecommunications)	70

3.9.4	Tier IV (Telecommunications)	70

3.10	Hierarchy Design Overview Data Center	71

3.10.1	Entrance Facility	73

3.10.2	Main Distribution Area (MDA)	74

3.10.3	Horizontal Distribution Area (HDA)	76

3.10.4	Storage Area Network (SAN)	77

3.10.5	Access Floor	77

3.10.6	GSM Landing Point	78

3.10.7	Cable Containment	78

3.10.8	Utility Services	78

3.10.9	Environmental Requirements	78

3.10.10	Electromagnetic Interference	79

3.10.11	Lighting Requirements	79

3.10.12	Fire Suppression System	79

3.10.13	Video Surveillance Monitoring for DC and TER	79

3.10.14	Access Control for DC and TER Rooms	79

3.10.15	Electrical Systems	80

3.10.16	Earthing Requirements	80

3.10.17	Labeling	80

3.10.18	Design Considerations for Skolkovo	80

3.10.19	Estimate for Data Center Power Requirements	82

3.10.20	Estimate for Primary PoP Power Requirements	84

3.10.21	Estimate for Secondary PoP Power Requirements	87

4	Appendix A – Blown Fibre	90

5	Appendix B – ITU G.652.D Fiber Optic Cable	91

6	Appendix C – LC Fiber Optic Connector	93

7	Appendix D – Fiber Optic Cabling Splicing	94

8	Appendix E – Cabling Standards	98

9	99

9	Glossary	100

10	About This Document	105

10.1	History	105

10.2	Review	105

10.3	Document References	105

11	Document Acceptance	106

Figures

Figure 1 – Active Network Conceptual Design	15

Figure 2 –Example of a 12 Way Duct Elevation	16

Figure 3 – Example of Manhole Details	18

Figure 4 – Sectional Elevation of JRC-14 Manhole	18

Figure 5 – Sectional Elevation of MR-810A Manhole	19

Figure 6 – Half Sectional View of MRT-4A Manhole	20

Figure 7 – Half Sectional View of MRT-4A Manhole	20

Figure 8 – Graph of Signal Loss against Wavelength for Multiple Fibre Types	24

Figure 9 – High Density Fibre Splice Patch Panel	27

Figure 10 – Spice Tray	27

Figure 11 –Low Profile, High Density Fibre Patch Panel	29

Figure 12 – Example of a LC Connector	30

Figure 13 – LC Connector Performance Chart	30

Figure 14 – Architecture of Intelligent Infrastructure Management	33

Figure 15 – LCD Screen on Intelligent Patch Panel	33

Figure 16 – Intelligent Fibre Patch Panel	34

Figure 17 – Example of ONMS Components	36

Figure 18 – Reflectometric Trace Link	38

Figure 19 – OSP View – Drawing Format	39

Figure 20 – OSP View – Ariel Photo	40

Figure 21 – Duct Bank Detail for Under Ground Deployment	44

Figure 22 – Duct Bank Detail for Bridges	45

Figure 23 – Inner Ducts	46

Figure 24 – Centre Conduit Entry in Manhole	48

Figure 25 – Typical Communications Cabling Manhole	49

Figure 26 – Splay Conduit Entry in Manhole as an Alternative Option	49

Figure 27 – Typical Handhole left and Duct Egress (above right)	50

Figure 28 - Pre moulded Handhole	50

Figure 29 – Correct and Incorrect Duct Entry/Exit from a Manhole	51

Figure 30 – A 12 and 6 Way Ductbank	52

Figure 31 – A 4 Way Ductbank	53

Figure 32 – Pictorial View of a Splice Enclosure	54

Figure 33 – Mid-Span Access at Multiple Plots in MDU & Commercial/Industrial Areas	55

Figure 34 – Midspan Splicing Details	56

Figure 35 – Last Mile Direct Fibre (Non-diverse) for SDU Plots	57

Figure 36 – Pictorial Representation of Midspan Splicing Activity in Field	58

Figure 37 – Power Separation Guidelines – 1	63

Figure 38 – Power Separation Guidelines – 2	64

Figure 39 – Hierarchy Design Overview	72

Figure 40 – Example Data Center	73

Figure 41 – Example MDA	75

Figure 42 – Example HDA	76

Figure 43 – Example SAN	77

Figure 44 – Fibre Cleaver View	94

Figure 45 – Fibre Alignment View	95

Figure 46 – Fibre Alignment Magnified View	95

Figure 47 – Splicing Electrodes View	96

Figure 48 – Fibre Splice Protective Sleeve	97

Tables

Table 2-1 – Different Proposed Sizes of Tertiary POPs	12

Table 2-2 – Internal Manhole Dimensions	17

Table 2-3 – Last Mile Fiber Core Count Recommendation	23

Table 2-4 – Fibre Colour Code for Splicing	28

Table 2-5 – Duct Sizes for Different Roads	44

Table 2-6 – Inner-duct Sizes	46

Table 2-7 – Duct Bank Configuration	52

Table 2-8 – Duct Profile at Building	59

Table 3-1 – Power Separation Distances	62

Table 3-2 – Tray Size Capacities	65

Table 3-3 – Tray Size Capacities	66

Table 3-4 – Equipment Room Sizes	67

Table 3-5 – Example Data Centre Power Requirements	83

Table 3-6 – Example Primary POP Power Requirements	85

Table 3-7 – Example Secondary POP Power Requirements	88

Table 5-1 – G.652.D Fibre Specification	92

Table 6-1 – LC Connector Performance	93

Документ №6:

Table of Content

1	Executive Summary	7

2	Document Objectives	10

3	Convergence Design Guideline	11

3.1	Value Added Services Creation	11

3.1.1	System Integration and Service Creation	12

3.1.2	Technological Platforms	14

3.1.3	Converged Network	15

3.2	Convergence Architecture	17

3.2.1	Development Level	17

3.2.2	City Level Approach	17

3.2.3	Building Level Approach	19

3.3	Convergence Benefits	23

3.3.1	Skolkovo	23

3.3.2	Occupants/tenants	23

3.3.3	Design Engineers	24

3.3.4	Contractors	24

3.3.5	Manufacturers	24

3.4	Integrated Operation Centre Concept	25

3.4.1	Overview	25

3.4.2	IOC Benefits	26

4	Building & ICT Systems Basis of Design	28

4.1	HVAC Control	29

4.1.1	System Description	29

4.1.2	System Components	30

4.1.3	Specific Technical Requirements	32

4.2	Lighting Control	35

4.2.1	Description	35

4.2.2	System Components	35

4.2.3	Specific Technical Requirements	37

4.3	Metering	40

4.3.1	System Description	40

4.3.2	System Components	40

4.3.3	Specific Technical Requirements	41

4.4	Access Control	44

4.4.1	Description	44

4.4.2	System Components	45

4.4.3	Specific Technical Requirements	46

4.5	Video Surveillance	51

4.5.1	Description	51

4.5.2	System Components	52

4.5.3	Specific Technical Requirements	53

4.5.4	Bandwidth Consideration	57

4.6	Car Parking	58

4.6.1	Description	58

4.6.2	System Components	58

4.7	Digital Signage	63

4.7.1	Description	63

4.7.2	System Components	63

4.7.3	Specific Technical Requirements	65

4.8	Audio/Video System	69

4.8.1	Description	69

4.8.2	System Components	69

4.8.3	Specific Technical Requirements	71

4.9	Tracking System	75

4.9.1	Description	75

4.9.2	System Components	75

4.9.3	Specific Technical Requirements	77

4.10	Voice (Phone) Service	81

4.10.1	Voice Architecture	81

4.10.2	Contact Centre	83

4.11	Voice, Video and Data Network	86

4.12	IP Network Security	87

4.13	IP Network Management	88

4.14	Wireless	89

4.15	Structured Cabling	91

4.16	Others	92

4.16.1	Life Safety	92

4.16.2	Elevators and escalators	92

4.16.3	Uninterruptible Power Supply (UPS)	92

4.16.4	Public Address / Back Ground Music (PA/BGM)	93

5	Appendix-A	94

6	Appendix-B	98

7	Appendix-C	99

8	Appendix-D	106

9	Appendix-E	113

10	Appendix-F	117

11	Appendix-G	121

12	Glossary	124

13	About This Document	127

13.1	Review History	127

13.2	Document References	127

14	Document Acceptance	128

Table of Figures

Figure 1: Skolkovo’s Connected Real Estate Framework	8

Figure 2: Skolkovo’s Connected Real Estate General Architecture	9

Figure 3:  Value-Added Service Creation Approach	10

Figure 4: Value Added Service Creation Framework	11

Figure 5: Service Delivery Platform	13

Figure 6: CRE Building Architecture	15

Figure 7: Skolkovo’s City Converged Approach	18

Figure 8: Skolkovo’s Building Converged Approach	20

Figure 9: Typical Building System Connectivity	21

Figure 10: IOC Overview	26

Figure 11: HVAC Control Single Line Diagram	33

Figure 12: Lighting Control Single Line Diagram	38

Figure 13: Metering Single Line Diagram	42

Figure 14: Access Control System Schematic	45

Figure 15: Access Control Single Line Diagram	48

Figure 16: IP Video Surveillance System	51

Figure 17: Video Surveillance Single Line Diagram	55

Figure 18: Car Parking Single Line Diagram	60

Figure 19: Hardware Manager	64

Figure 20: Digital Media Player	64

Figure 21: Typical Digital Signage Network Topology	65

Figure 21: Digital Signage Single Line Diagram	67

Figure 22: Touch Panel	69

Figure 23: Telepresence	70

Figure 24: Audio Video Single Line Diagram	72

Figure 26: Audio Video Single Line Diagram	73

Figure 25: Passive RFID Tag	75

Figure 26: RFID Reader	76

Figure 27: Asset Tracking Single Line Diagram	78

Figure 28: Conceptual Design of Voice Service Provider Network	81

Figure 29: Contact Centre Architectural View	84

Figure 30: Security Process Wheel	87

Figure 31: Mobility High Level Architecture	89

Figure 32: IP Thermostat	95

Figure 33: Gateway	96

Figure 34: Sample of Contact Smart Card	100

Figure 35: Sample of Contactless Smart Card	101

Figure 36: Door Access Controller	102

Figure 37: Optical Portals	103

Figure 38: Turnstile	104

Figure 39: Sample Biometric Readers	105

Figure 40: IP Camera	106

Figure 41: UTP Cables	107

Figure 42: Cables Comparison	108

Figure 43: Screen Display	109

Figure 44: ANPR Camera	115

Figure 45: Handheld Monitor	116

Figure 46: Bollard System	116

Figure 47: LCD Display and Kiosk	117

Figure 48: PDP Display	118

Figure 49: LED Panel	118

Figure 50: Single Encoder	118

Figure 51: Multi-channel Encoder	118

Figure 52: Cables and Connectors	119

Figure 56: Projector	122

Документ №7:

Table of Contents

1	Introduction	5

1.1	Purpose	5

1.2	Services Strategy Development	6

1.3	Audience	7

1.4	Scope	8

1.5	Assumptions	8

2	Overview	10

2.1	Classifications	11

2.1.1	Tier 1 Data Center – Basic	11

2.1.2	Tier 2 Data Center – Redundant Components	11

2.1.3	Tier 3 Data Center – Concurrently Maintainable	11

2.1.4	Tier 4 Data Center – Fault Tolerant	12

2.2	Telecommunication Tier Types	12

2.2.1	Tier 1 (Telecommunications)	12

2.2.2	Tier 2 (Telecommunications)	13

2.2.3	Tier 3 (Telecommunications)	15

2.2.4	Tier 4 (Telecommunications)	16

2.3	Hierarchical Design	18

2.3.1	Entrance Facility	20

2.3.2	Horizontal Distribution Area (HDA)	21

2.3.3	Storage Area Network (SAN)	22

2.3.4	Access Floor	23

2.3.5	GSM Landing Point	23

2.3.6	Cable Containment	23

2.3.7	Utility Services to be avoided	23

2.3.8	Environmental Requirements	23

2.3.9	Electromagnetic Interference	24

2.3.10	Lighting Requirements	24

2.3.11	Fire Suppression	24

2.3.12	Video Surveillance	24

2.3.13	Access Control	25

2.3.14	Electrical Systems	25

2.3.15	Earthing Requirements	25

2.3.16	Labeling	25

3	Design Considerations	26

3.1	Space Requirements	27

3.2	Power Requirements	28

4	Technical Architecture	29

4.1	Functions	29

4.2	Infrastructure	30

4.2.1	Core	34

4.2.2	Aggregation	35

4.2.3	Access	39

4.2.4	Storage	44

4.2.5	Virtualization – Storage, Computing Power and Security	46

4.2.6	Distributed Design Topology and Redundancy	49

4.2.7	Network Management	54

4.2.8	Testing	56

4.2.9	Capacity Planning and Growth	56

5	Appendix 1	57

6	Glossary	58

7	About This Document	61

7.1	History	61

7.2	Document References	61

8	Document Acceptance	61

Table of Figures

Figure 1-1: Skolkovo Overall ICT Technical Architecture	5

Figure 1-2: Layered Services Approach	7

Figure 2-1: Typical Tier 1 facility layout	13

Figure 2-2:Typical Tier 2 facility layout	14

Figure 2-3: Typical Tier 3 facility layout	16

Figure 2-4: Typical Tier 4 facility layout	17

Figure 2-5: Hierarchical Design Overview	18

Figure 2-6: Data Center Example	19

Figure 2-7: MDA Example	21

Figure 2-9: HDA Example	22

Figure 4-1: Skolkovo DC Functional Blocks	30

Figure 4-2: Data Center Infrastructure Tiered Architecture	31

Figure 4-4: Data Center Infrastructure Tiered Architecture	33

Figure 4-5:  Logical Flow Traffic through DC Core	35

Figure 4-6: Aggregation Block	36

Figure 4-7:  Virtual Device Context	37

Figure 4-8: Load Balancing Appliance Logical Flow	38

Figure 4-9 – Firewall Appliance Logical Flow	39

Figure 4-10 – Legacy Data Center Multi Port Model	40

Figure 4-11 – Legacy and Unified Server Connectivity Models	41

Figure 4-12 – Fiber Channel over Ethernet (FCoE)	41

Figure 4-13 – Unified I/O Design – Option 1	42

Figure 4-14 – Unified I/O Design – Option 2	43

Figure 4-15 – Different Network Islands Virtualized To Common Data Center Infrastructure	47

Figure 4-16 – Proposed Design Showing Options for DR	50

Figure 4-17 – DR Solution using GSS	52

Figure 4-18 – Global Site Selection Process	53

List of Tables

Table 1 – Power Requirements Estimates	29

Table 2: Budgetary Cost Estimates	57